# CSE421 Project: Genome Assembly Research Network

Below is a comprehensive, structured documentation of the physical design, hardware adaptations, cabling architecture, and base IP configurations implemented so far [1, 2].

## Section 1: Student ID to Base Network Conversion

To establish a unique addressing scheme, the base IPv4 address block was derived directly from the student IDs of the first two group members:

- Member 1:

  - Student ID = 22101569 → Last two digits = 69 (assigned as the first octet).

- Member 2:

  - Student ID = 22101844 → Last two digits = 44 (assigned as the second octet).

- Base Network Block: 69.44.0.0/16

This /16 block was then subnetted using Variable Length Subnet Masking (VLSM) to accommodate the host requirements of the six distinct research units and their interconnecting WAN links, optimizing address space efficiency [1, 2].

## Section 2: Physical Topology & Hardware Adaptations

To represent each of the six research units, Cisco 2911 Integrated Services Routers (ISRs) were deployed. Because the 2911 router natively features only three onboard GigabitEthernet ports, physical modifications were required to support multiple point-to-point connections:

#### 1. Hardware Expansion (Serial Modules)

- The Problem: The central Sequencing Core Lab (SCL) and the Genome Validation Unit (GVU) require more physical ports than are natively available to connect to multiple neighboring routers and local LANs simultaneously.

- The Solution: The routers were powered down, and two HWIC-2T (High-Speed WAN Interface Card with 2 Serial Ports) modules were physically installed into the expansion slots of every router. This provides up to four Serial ports per device, ensuring full physical scalability for both primary and backup paths.

#### 2. Slot Mapping Observation (Physical Slot Assignment)

- During the hardware installation, the modules were populated into Slot 2 and Slot 3 of the chassis.

- This physical placement dynamically designated the interface identifiers in the Cisco IOS software as Serial0/2/0, Serial0/2/1, Serial0/3/0, and Serial0/3/1.

- This slot mapping was carefully noted to ensure that all subsequent CLI configuration scripts accurately targeted the physical ports on the devices.

## Section 3: Cabling and Infrastructure Design

The physical connections across the topology were divided into three architectural layers:

#### 1. Local Area Networks (LANs)

- Medium: Copper Straight-Through cables.

- Design: Each research unit LAN consists of end-devices (PCs and a Printer) connected to a Cisco Catalyst 2960 24-Port Switch.

- Gateway Connection: To maintain configuration uniformity across all units, the local switch is connected to the router's GigabitEthernet0/0 port. This interface acts as the default gateway for the entire LAN.

- SCL LAN Expansion: SCL hosts the core servers (DNS, Web, Email, and DHCP), which are connected directly to SCL's local switch (Switch0).

- MDU LAN Expansion: MDU hosts its own local DHCP and Email servers connected to Switch3.

#### 2. The Shared Backbone WAN

- Medium: Copper Straight-Through cables.

- Design: As required, the Read Mapping Unit (RMU), De Bruijn Graph Unit (DBG), and Motif Discovery Unit (MDU) are connected through a shared backbone switch (Switch6).

- Routing Method: Rather than point-to-point links, this multi-access WAN operates within a single subnet (69.44.8.192/29). Routers connect to the backbone switch via their GigabitEthernet0/1 interfaces.

#### 3. Point-to-Point WAN Links

- Medium: Serial DCE cables.

- Design: High-speed point-to-point serial links were established between neighboring routers.

- Clocking: SCL and GVU act as the DCE ends for their respective links, meaning clock rate 64000 was configured on their interfaces to synchronize data transmission across the physical medium.

## Section 4: Gateway & Static IP Address Mapping

A strict IP allocation strategy was applied to static network elements [2]:

- Default Gateways: The first usable host IP address of each LAN subnet was consistently assigned to the router's GigabitEthernet0/0 interface.

- Servers & Printers: Configured with static IP addresses starting from the second usable address upward (e.g., .2, .3, .4).

- Archive Storage Node (ASN): As ASN is a static-only environment, both PC10 and PC11 were configured manually with static IPs, subnet masks, default gateways, and a pointer to the SCL DNS Server (69.44.0.2).

#### Summary of Gateways and Dedicated Device IPs:

| Device Name | Assigned IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| SCL Router (LAN Gateway) | 69.44.0.1 | 255.255.252.0 | N/A |
| - SCL DNS Server | 69.44.0.2 | 255.255.252.0 | 69.44.0.1 |
| - SCL Web Server | 69.44.0.3 | 255.255.252.0 | 69.44.0.1 |
| - SCL Email Server | 69.44.0.4 | 255.255.252.0 | 69.44.0.1 |
| - SCL DHCP Server | 69.44.0.5 | 255.255.252.0 | 69.44.0.1 |
| - SCL Printer (Printer0) | 69.44.0.6 | 255.255.252.0 | 69.44.0.1 |
| RMU Router (LAN Gateway) | 69.44.4.1 | 255.255.254.0 | N/A |
| - RMU Printer (Printer1) | 69.44.4.2 | 255.255.254.0 | 69.44.4.1 |
| DBG Router (LAN Gateway) | 69.44.6.1 | 255.255.255.0 | N/A |
| - DBG Printer (Printer2) | 69.44.6.2 | 255.255.255.0 | 69.44.6.1 |
| MDU Router (LAN Gateway) | 69.44.7.1 | 255.255.255.0 | N/A |
| - MDU Email Server | 69.44.7.2 | 255.255.255.0 | 69.44.7.1 |
| - MDU DHCP Server | 69.44.7.3 | 255.255.255.0 | 69.44.7.1 |
| - MDU Printer (Printer3) | 69.44.7.4 | 255.255.255.0 | 69.44.7.1 |
| GVU Router (LAN Gateway) | 69.44.8.1 | 255.255.255.128 | N/A |
| - GVU Printer (Printer4) | 69.44.8.2 | 255.255.255.128 | 69.44.8.1 |
| ASN Router (LAN Gateway) | 69.44.8.129 | 255.255.255.192 | N/A |
| - ASN Printer (Printer5) | 69.44.8.130 | 255.255.255.192 | 69.44.8.129 |
| - ASN Client PC (PC10) | 69.44.8.131 | 255.255.255.192 | 69.44.8.129 |
| - ASN Client PC (PC11) | 69.44.8.132 | 255.255.255.192 | 69.44.8.129 |

## Section 5: Current Implementation Status

- Physical Topology: All routers, switches, servers, printers, and client PCs have been successfully placed, module-expanded, and interconnected.

- Interface Initialization: Basic configuration scripts have been executed on all six routers. This process:

  - Set custom hostnames on each router corresponding to its research unit.

  - Disabled DNS domain-lookup on routers to prevent input freezes during CLI typos.

  - Assigned static IP addresses and subnet masks to all local LAN interfaces and interconnecting WAN interfaces.

  - Issued the no shutdown command on all configured interfaces, successfully bringing up all links to an active state (indicated by green connection triangles across the entire topology).

- Static Endpoint Configuration: All servers, printers, and the ASN LAN end-devices have been configured manually with their respective IP parameters, establishing physical and local-link readiness.

## Section 6: LAN Sub-nets (Sorted from Largest to Smallest)

| Unit Name | Required Hosts | Subnet Mask | CIDR | Network Address | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- |
| Sequencing Core Lab (SCL) | 520 | 255.255.252.0 | /22 | 69.44.0.0 | 69.44.0.1 - 69.44.3.254 | 69.44.3.255 |
| Read Mapping Unit (RMU) | 300 | 255.255.254.0 | /23 | 69.44.4.0 | 69.44.4.1 - 69.44.5.254 | 69.44.5.255 |
| De Bruijn Graph Unit (DBG) | 220 | 255.255.255.0 | /24 | 69.44.6.0 | 69.44.6.1 - 69.44.6.254 | 69.44.6.255 |
| Motif Discovery Unit (MDU) | 160 | 255.255.255.0 | /24 | 69.44.7.0 | 69.44.7.1 - 69.44.7.254 | 69.44.7.255 |
| Genome Validation Unit (GVU) | 90 | 255.255.255.128 | /25 | 69.44.8.0 | 69.44.8.1 - 69.44.8.126 | 69.44.8.127 |
| Archive Storage Node (ASN) | 45 | 255.255.255.192 | /26 | 69.44.8.128 | 69.44.8.129 - 69.44.8.190 | 69.44.8.191 |

## Section 7: WAN Sub-nets (Connections between Routers)

| WAN Link | Required IPs | Subnet Mask | CIDR | Network Address | Usable IP Range |
| --- | --- | --- | --- | --- | --- |
| Shared Backbone (RMU-DBG-MDU) | 3 | 255.255.255.248 | /29 | 69.44.8.192 | 69.44.8.193 - 69.44.8.198 |
| SCL to RMU | 2 | 255.255.255.252 | /30 | 69.44.8.200 | 69.44.8.201 - 69.44.8.202 |
| SCL to DBG | 2 | 255.255.255.252 | /30 | 69.44.8.204 | 69.44.8.205 - 69.44.8.206 |
| SCL to GVU | 2 | 255.255.255.252 | /30 | 69.44.8.208 | 69.44.8.209 - 69.44.8.210 |
| MDU to GVU | 2 | 255.255.255.252 | /30 | 69.44.8.212 | 69.44.8.213 - 69.44.8.214 |
| DBG to MDU (Backup Link) | 2 | 255.255.255.252 | /30 | 69.44.8.216 | 69.44.8.217 - 69.44.8.218 |
| ASN to GVU | 2 | 255.255.255.252 | /30 | 69.44.8.220 | 69.44.8.221 - 69.44.8.222 |